### Norstar Telephone Feature Guide        KSU Modular DR5

Contrast Adjustment                FEATURE * 7

Adjust the contrast of your display.

Press FEATURE * 7, then press a number from 1 to 4 (depending on your telephone). 	Press HOLD to set your choice

Using a Norstar Features:

Press FEATURE and enter the desired feature code on the dial pad.

or    press a programmed memory button.

Follow the display messages.  On a two-line display telephone, some features are also

available on display buttons.

=========================================================================== Programming Memory Buttons

You can program memory buttons for one-touch access to frequently used features

Press FEATURE *3.

Press the memory button that you want to program.

Press FEATURE and enter the feature code on the dial pad.

You can also program frequently-dialed numbers, using the following codes:

FEATURE *1  External autodial:    Store an external number for one-touch dialing.

FEATURE *2   Internal autodial:    Store an internal number for one-touch dialing.

You cannot program line, intercom  or Handsfree/Mute bottons.

### ===========================================================================

### Norstar Features

Background Music         FEATURE 86 (On)                             FEATURE #86(Off)

Listen to music (provided by your office) through your telephone speaker when you are not 	on a call.

Button Inquiry                 FEATURE * 0

Check what is programmed on any button. Use when labeling buttons.

1  Press     FEATURE * 0

2  Press button you want  to check

Call Duration Timer        FEATURE 77

Briefly display the approximate length of your current or most recent call.

Call Forward                    FEATURE 4(On)                               FEATURE #4(Off)

Send all your calls to another telephone in your Norstar system.

1 Press          FEATURE 4

2 Dial the phone to which the calls are to be sent ( or press Inside Autodial )

Call Park                           FEATURE 74

Put a call on hold so that it can be picked up from any telephone in your Norstar system. 	The display shows a three-digit retrieval code.

To retrieve a parked call, press pbi and dial a retrieval code on any telephone in your

Norstar  system.

Call Pickup-Group           FEATURE 75

Answer a call that is ringing at another telephone in your pickup group.

The external call that has been ringing longest is answered first.

Conference                        FEATURE 3

Establish a conference call between yourself and two other parties.

1   Make or answer the first call.

2   Put the first call on hold.

3   Make or answer the second call.

4   After the second call is connected, press FEATURE 3.

5   Press the line or intercom button of the first held call

6   Press RLS to end the conference call.

To remove yourself from a conference call permanently (unsupervised conference)

Press FEATURE 70. The other two callers remain connected. (Some external lines may not 	support this feature. See your system coordinator.)

To put a conference on hold

Press HOLD. The other two callers can still talk to each other.

To split a conference

Press the line or intercom button of the caller you want to disconnect, then press Rls.

Press the line or intercom button of the remaining caller to resume your conversation.

To independently hold two calls

Press the line or intercom button of the first caller, then press HOLD. The second caller

is automatically put on hold. To re-establish the conference, retrieve one call from hold, 	press FEATURE 3, then retrieve the second call from hold.

Contrast Adjustment         FEATURE * 7

Adjust the contrast of your display.

Press FEATURE * 7, then press a number from 1 to 8 (depending on your telephone). 	Press HOLD to set your choice.

Dialing Modes                  FEATURE * 82

Choose one of the three methods of dialing.

1  Press   FEATURE * 82

2  Press    #              to select the mode

3  Press   HOLD       to store the mode.

Standard Dial: 	Select a line, then dial the number. (Standard Dial is always available, even 			when another dialing mode is selected.)

Automatic Dial: Dial the number without choosing a line button first. Your prime line is 				automatically selected for the call.

Pre-Dial:  Dial the number, then press a line button to place the call. Edit the number

by  pressing VOLUME BAR before placing the call.

Do Not Distrub              FEATURE 85    (On) ,             Cancel    FEATURE # 85    (Off)

When you are not on a call prevent all incoming calls, except priority calls, 	from  			ringing at your telephone. When you are on a call, block an incoming priority call.

Hold                               HOLD

Temporarily suspend a call.    ( Others can pick it up )

To retrieve a held call, press the line button for the held call.

Exclusive Hold                FEATURE 79  or    FEATURE HOLD

Temporarily suspend a call and prevent other telephones from picking it up.

Language Choice            FEATURE * 501

Select English as the language for the telephone display.

FEATURE * 502

Select alternate language for the telephone display.

Last Number Redial         FEATURE 5

Automatically redial the last external telephone number that you dialed.

Line Pools                        FEATURE 64

With a line pool, telephones can share several lines for making calls.

Press FEATURE * 64  or  INTERCOM.

Enter a line pool access code.  (See your system coordinator for a list.)

Feature available only when line pool access is set up

Link                                    FEATURE 71

Generate a FLASH signal

Messages                           FEATURE 1 (On)                  FEATURE # 1 (Off)

Send a message to a Norstar telephone's display to have someone call you back.

To view and reply to your messages

1  Press   FEATURE 65

2  Press  * and  #  to view your message list.

3  Press 0   to call the person who left you the message.

To erase a message

Press HOLD while viewing a message.

Name and Number Blocking    FEATURE 819 (On)      Cancel          FEATURE #819 (Off)

Block the outgoing name and/or number on a per-call basis.

Page                                        FEATURE 60  and code (1 to 3) and zone (0 to 3)

Make a page announcement through either the internal (code 1) or external (code 2) speakers, or both (code 3). Zone 0 pages all zones.

Page announcements are programmed to timeout after a pre-selected amount of time which is set by your installer.

Internal Page                           FEATURE 61 and zone (0 to 3)

Make a page announcement to all, or to a specific group to Norstar telephones, through the telephone speakers. Zone 0 pages all zones.

External Page                         FEATURE 62 and zone (0 to 3)

Make a page announcement through an external loudspeaker system.

Internal and External Page   FEATURE 63 and zone (0 to 3)

Make a page announcement through both your Norstar telephones and an external loudspeaker system. Zone 0 pages all zones.

Pause                                     FEATURE 78

Program in an external autodial sequence to insert a 1.5 second delay .

For pulsing dialing: *  also inserts a 1.5 second delay.

Priority                                 FEATURE 69

Interrupt a person who is on call or using Do Not Disturb.

A person on another call can press FEATURE 85 to block your priority call.

Ring Again                           FEATURE 2

Monitor a busy or unanswered telephone, or a busy line pool within your system. Ring Again signals you to call back when the telephone or line pool becomes available.

Ring Type                             FEATURE * 6

Select a distinctive ring to help differentiate between your telephone and others nearby.

1   Press FEATURE *  6

2   Enter the ring type number (1 to 4)

3   Press HOLD.

Ring                                     FEATURE *80

Make your telephone ring so that you can adjust the volume. You can also adjust the volume any time your telephone rings.

Run/Stop                              FEATURE * 9

Store most than one autodial number or external carrier feature code on one memory button by inserting a break point  between numbers or codes. The first press of the button dials the first number or code; the next press dials the next number or code. You can program up to four numbers or codes separately by break points.

Saved Number Redial          FEATURE 67

Save a number to redial later. Enter the code while you are on a call that you have dialed to save the number. Enter the code when you are not on a call to redial the saved number.

Speed Dial                                FEATURE 0

Dial an extension telephone number using a two-digit code. There are two types of speed

dial codes: system (01 to 70) and personal (71 to 94). System speed dial codes can be

used from any Norstar telephone in the system. They are assigned by your system coordinator. Personal speed dial codes are used exclusively at your telephone.

To make a call using a speed dial code

1   Press FEATURE 0

2   Enter the two-digit code for the number.(01 to 70 for system speed dial,

71 to 94 for personal speed dial

To program personal speed dial numbers

1	Press FEATURE * 4

2   Enter a two-digit code from 71 to 94

3   Specify the external line by pressing a line button, or the intercom button.

If you don't specify the external line, the system automatically chooses

a line  for the call.

4   Dial the telephone number you want to program (up to 16 digits).

5   Press HOLD.

6   Record the code and number you have just programmed.

Static Time and Date    FEATURE 806 (On)            FEATURE # 806(Off)

Change the first line of the display to the current time and date.

Time                                FEATURE 803

Briefly display the time and date while you are on a call.

Transfer                           FEATURE 70

Send a call to another telephone within your Norstar system, or to an external 			telephone.  You may not be able to transfer a call on an external line to

an external telephone, depending on the capabilities of the lines.

1   Make or answer the call.

2   Press FEATURE 70.

3   Call the person you want to transfer the call to.

4   Stay on the line if you wish to speak to the person first.

5   Press  Rls  to complete the transfer.

If an external call is transfered to a busy internal or network extension,

or is not answered after a few rings, the call automatically rings you back.

Trunk Answer                 FEATURE 800

Answer an external call that is ringing on a line that has been placed into

a Ringing Service  schedule from any telephone in your Norstar system.

This feature does not work for a private line.

Voice Call                         FEATURE 66

Make a voice announcement or begin a conversation through the speaker

of another  Norstar telephone without first making the other telephone ring.

Voice Call Deny                FEATURE 88 (On)            FEATURE # 88 (Off)

Prevent your telephone from receiving voice calls.

Do Not Disturb  ( FEATURE 85 ) also prevents your telephone from receiving voice calls.

Wait for Dial Tone             FEATURE 804

Program in an external autodial number to cause the system to wait to receive dial tone  	from another system before proceeding with the dialing sequence.